Strive to create an intelligent life

[image: image33.jpg]~=—30,0000 —=

|

30.0000

¢





catalogue

Product parameters and specifications
4
Product hardware introduction
5
A-ES801 Schematic diagram of panel
5
A-ES802 Schematic diagram of panel
6
A- ES827 Schematic diagram of panel
7
A-ES832 Schematic diagram of panel
8
ST881 lock body schematic diagram
9
ST882 lock body schematic diagram
10
ST883 lock body schematic diagram
11
Product installation tips
12
Product operation instructions
13
Local operation instructions
13
Light indication
13
Register the fingerprint
14
Add a new fingerprint
15
Use of Morse password:
15
Product standards and packaging specifications
18
A-ES801Supporting standards
18
A-ES801packing specifications
18
A-ES802Supporting standards
19
A-ES802 packing specifications
19
A-ES827 Supporting standards
20
A-ES827packing specifications
20
A-E832 Supporting standards
21
A-ES832 packing specifications
21
Product parameters and specifications

	A-ES800Table of series product specifications

	Support unlocking method
	Fingerprint

	Support number of users
	Fingerprint user: 20

	Local management function
	support

	power supply mode
	3 # 7 alkaline batteries

	Standby current
	Normal use can last for more than a year

	working current
	<50mA

	Standby power interface
	TYPE-C USB

	working temperature
	-20°C ~ 60°C

	relative humidity
	20%~ 85% (non-condensation)

	Antistatic capacity (ESD)
	Contact discharge  ± 8KV, non-connected discharge  ± 15KV

	Recommended usage scenarios
	Wardrobe / bedside table / shoe cabinet / drawers, etc


Product hardware introduction

A-ES801 Schematic diagram of panel

[image: image34.jpg]



When installing the panel, the opening of the panel is shown in the figure below, with a 16mm diameter mounting hole in the middle.

[image: image35.jpg]


A-ES801 Comes with 16mm panel screws that can be matched, with a thickness of 15mm~15.50mm panel.

A-ES802 Schematic diagram of panel【Discontinued】

[image: image36.jpg]



[image: image37.jpg]


When installing the panel, the opening of the panel is shown in the figure below, with a 16mm diameter mounting hole in the middle.

A-ES802 Comes with 16mm panel screws that can be matched, with a thickness of 15mm~15.50mm panel.

A-ES827 Schematic diagram of panel

Main schematic diagram

[image: image38.jpg]



[image: image39.jpg]55

@a50_

45

12.10

7.10




When installing the panel, the opening of the panel is shown in the following figure, with a mounting hole of 30.5mm diameter opened in the middle.

A-ES827 Attached with 14mm*3mm;15.70mm * 3mm panel screws, matching, 15mm~25mm thickness panel.

A-ES832 Schematic diagram of panel

Main schematic diagram

[image: image40.jpg]Sl

191

54

SL

80l





[image: image41.jpg]Received
product

Test setup
product

opening

Installing the
front panel

Installing lock
buckles

Installing lock
body




When installing the panel, the panel opening is shown in the following figure, with a 10mm diameter installation hole in the middle.

A-ES832 Comes with 16mm panel screws that can be matched, with a panel thickness of 15mm~30mm.

ST881 lock body schematic diagram【Discontinued】

Main schematic diagram

[image: image42.png]



[image: image43.png]


Schematic diagram of buckle

ST882 lock body schematic diagram

[image: image44.png]


Main schematic diagram

Schematic diagram of buckle

SHAPE  \* MERGEFORMAT



ST883 lock body schematic diagram

Main schematic diagram

[image: image45.jpg]



[image: image46.jpg]


Schematic diagram of buckle

Product installation tips

It is recommended to install this product in the following steps

[image: image47.jpg]



matters need attention:

After receiving the product, it is suggested to install the battery and connect the line before installation, and simply test the basic functions of the product, including whether the lock tongue is in and out normally, whether the fingerprint can be normally input normally, and whether the Bluetooth is connected normally.

It is suggested to set up, then install, so as to avoid the installation, found problems waste installation time.

It is suggested to install the buckle first, and then install the lock body, and then according to the position of the buckle, the installation of the lock body, the buckle is similar to hidden installation, in case of installation problems, need to re-install, does not affect the appearance of the product, if the position of the installation of the problem, reinstallation, may affect the appearance.

Product operation instructions

In order to facilitate you to use this product, to meet the habits of more people using the product, this product supports local management, let's introduce below.

Local operation instructions

This product, initialized to the default experience mode, "experience mode state, any fingerprint can be unlocked".

Light indication:

Fingerprint verification success: the indicator light is a blue light flash, indicating that the device is in the factory state
Fingerprint verification success: the indicator light is

green flash

Fingerprint verification failed: the indicator light is red flashing three times

Low power alarm: after successful fingerprint verification, the green light flashes, and the red light flashes three times means that the power is insufficient
Register the fingerprint:

Timtimeout in registration will quit, and you need to start again. Flash the red light in the registration.(Note: the first fingerprint registration, two fingerprints by default)

Press the finger to the green light (sky blue) and lift the finger;
Start to register your fingerprints;
2..3 Raise and press the finger ten times until the green light stops, indicating that the first fingerprint has been successfully registered;

2.4. After the first fingerprint is successfully registered, the blue light is on, and the second new finger is lifted and pressed ten times until the green light stops, indicating that the registration of the second fingerprint is successful. (Note: When inputting fingerprint, lift the finger once, do not press all the time. When recording, the finger should move up and down, left and right, and avoid pressing the same part of the finger ten times.）

Add a new fingerprint:

After verifying the input fingers, press the blue green light and lift it;

Start to input the new fingerprint, lift the finger and press it ten times until the green light stops, indicating that the registration of the new fingerprint is successful.

Use of Morse password: (After the restoration of factory Settings, all fingerprints cannot be restored.）

Use the Morse password to restore the factory settings  A, after the fingerprint is successful, press the blue green light (press do not lift);
B, The white light is on (long press do not lift);  C, until the red light is on and the green light is on, indicating that all fingerprints have been cleared and

the factory state is restored

Setting of the emergency  password

After unlocking the fingerprint, press the blue light (long press do not lift) until the white light is on;

After loosening the green light is on, press the fingerprint once (for example, press 5 is the number 5), press the set times and wait for the blue light to turn on;

C, After the blue light is on, press the set number of times (for example, press 6 is the number 6), press the set number of times and wait for the light to turn off. D.The number of the green light press and the number of the blue light press form two digits (say 56). Remember your password.

Use of the emergency  password

When the fingerprint cannot be unlocked normally, press with any fingerprint until the white light flashes and releases;

B, after the green light is released, press the set number of times (such as 5 times), and then wait for the blue light to come on;

C, The number of times the blue light is set (say 6), and

wait for the light to go  out.

D.If the red light flashes before the light is off, the password is wrong; if the green light flashes, the password is correct.

E, After the password is correct, the lock will automatically open, and the factory state will resume. The fingerprint needs to be registered again.

4.4 Notes se Code using  "th":

A, If the emergency unlock password is not set, the default password is 33. If it is set, the default password is invalid.

B. After the factory state is restored, the set password becomes invalid and becomes the default password.

C, the timeout will exit in the emergency unlock password setting and use, exit need to start again. Emergency unlocking password set timeout red light flash, emergency unlocking password use timeout red light flash twice.

Product standards and packaging specifications

A-ES801Supporting standards

A-E801+ST881 Supporting details (the lock body can be optional)

	graphic
	name
	size
	quantity

	[image: image2.jpg]



	A-ES801 face-plate
	30mm*30mm*8mm
	1

	[image: image3.jpg]2-9L5

IRE]





	ST883Lock body
	92mm*60mm*24 mm
	1

	[image: image4.jpg]



	lock catch
	55mm*21mm*12.

10mm
	1

	[image: image5.jpg]



	Panel fixed sheet
	35mm*16mm*1.2

mm
	1

	[image: image6.png]



	Cross-head wood screw (panel)
	3.5mm*16mm
	2

	[image: image7.png]



	Cross head wood screw (lock body)
	3.5mm*16mm
	2

	[image: image8.png]



	Cross-head wood screw (lock)
	3.5mm*16mm
	2

	[image: image9.png]



	Cross head tapping screw (fixed plate)
	KA2.2mm*12mm
	2


A-ES801packing specifications

A-ES801+ST883 packing specifications

	tare
	0.10kg
	Paper box size
	95 mm*95mm*25mm

	Boating standard
	100sets / 1 box; weight:24.5 kg
	Carton size
	47cm*38cm*34.5cm


A-ES802Supporting standards

A-ES802+ST881 Supporting details (the lock body is optional)

	graphic
	name
	size
	quantity

	[image: image10.jpg]




	A-ES802 face-plate
	31mm*31mm*6.8m m
	1

	[image: image11.jpg]



	ST881 Lock body
	94mm*35mm*18.5

mm
	1

	[image: image12.jpg]



	lock catch
	55mm*21mm*12.1

0mm
	1

	[image: image13.png]



	Cross-head wood screw (panel)
	3.5mm*16mm
	2

	[image: image14.png]



	Cross head wood screw (lock body)
	3.5mm*16mm
	2

	[image: image15.png]



	Cross-head wood screw (lock)
	3.5mm*16mm
	2


A-ES802 packing specifications

A-ES802+ST881 packing specifications

	tare
	0.10kg
	Paper box size
	115mm*77mm*50mm

	Boating standard
	72 sets / 1 box; weight: 6.7kg
	Carton size
	51cm*39cm*21.5cm


A-ES827 Supporting standards

A-ES827+ST882 Supporting details (the lock body is optional)

	graphic
	name
	size
	quantity

	[image: image16.jpg]



	A-ES827 panel
	35mm*30.5mm**12mm
	1

	[image: image17.png]



	ST882 Lock body
	108mm*54mm*19.5mm
	1

	[image: image18.jpg]



	lock catch
	45mm*19mm*14.5mm
	1

	[image: image19.jpg]Doono





	Panel fixed sheet
	36mm*16mm*1.10mm
	1

	[image: image20.jpg]




	Cross flat head screw (panel)
	11.5mm*3mm
	2

	[image: image21.jpg]



	Cross flat head screw (panel

backup)
	14mm*3mm
	2

	[image: image22.jpg]



	Cross flat head screw (panel

backup)
	15.70mm*3mm
	2

	[image: image23.png]




	Cross head wood screw (lock

body)
	3.5mm*16mm
	2

	[image: image24.png]




	Cross-head wood screw

(lock)
	3.5mm*16mm
	2

	[image: image25.png]



	Cross groove sunk wood screw
	2.8mm*13.8mm
	2

	[image: image26.png]



	Cross groove sunk wood screw
	2.5mm*8mm
	1


A-ES827packing specifications

A-ES827+ST882 pack

	tare
	0.15Kg
	Paper box size
	115mm*77mm*50mm

	Boating standard
	100 sets / 1 box; weight: 15.5kg
	Carton size
	46cm*37.5cm*34cm


A-E832 Supporting standards

A-ES832+ST881 Supporting details (the lock body is optional)

	graphic
	name
	size
	quantity

	[image: image27.jpg]




	A-ES832face-plate
	74.70mm*35mm*8.5m m
	1

	[image: image28.jpg]



	ST881 face-plate
	94mm*35mm*18.5mm
	1

	[image: image29.jpg]



	lock catch
	55mm*21mm*12.10mm
	1

	[image: image30.png]



	Locch cross tapping screw
	3mm*16mm
	2

	[image: image31.png]



	Lock-body cross self-tapping screw
	3mm*16mm
	2

	[image: image32.png]



	Panel cross with self-tapping screws
	3mm*16mm
	2


A-ES832 packing specifications

A-ES832+ST881 pack

	tare
	0.12Kg
	Paper box size
	115mm*77mm*50mm

	Boating standard
	72 sets / 1 box; weight: 9.4kg
	Carton size
	51cm*39cm*21.5cm


A-ES800 Series of single fingerprint cabinet lock


p r o d u   c t	m a n u a l   s





V e r s i o n  1 . 2 ,  J u l y ,  2 0 25





�








�








pilot lamp





�








�








�








Screw hole spacing





�








Battery warehouse





tapping hole





�








�








�











[image: image48.jpg]3

94

—
2
o
51,50 _ 05 _|
N

— Eke

B4R

12

18.50




[image: image49.jpg]B—43



[image: image50.jpg]| A
;
ALz |-
%5
B35 55
L1
982,15, *2-04,28" 5
[, B
pa= |
i A |
EW )
%
kd i
3

B-8| 125



[image: image51.jpg]


[image: image52.jpg]59.97

915
11.8,




[image: image53.jpg]238

6.9




[image: image54.jpg]


